MATLAB-BASED MODELLING OF VSB MODULATOR AND DEMODULATOR FOR ANOLOG TELEVISION COMMUNICATION APPLICATIONS

AIM:

i) To generate and analyse the Vestigial Sideband (VSB) modulated signal by performing DSB-SC modulation and applying a suitable filter using MATLAB.

Ii)  To implement coherent modulation and recover the original message signal from the VSB signal, and compare the transmitted and recoverd waveforms.

PROGRAM CODE:

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fc = 10000;       % Carrier frequency

fm = 1000;        % Message frequency

Ac = 1;           % Carrier amplitude

Am = 1;           % Message amplitude

% Message signal

m = Am*cos(2*pi*fm*t);

% Carrier signal

c = Ac*cos(2*pi*fc*t);

% AM signal

am = (1 + m).*c;

% VSB filtering

[b,a] = butter(6, [0.08 0.22], 'bandpass');

vsb = filter(b,a,am);

% Demodulation

demod = vsb .* (2*c);

% Low pass filtering

[b2,a2] = butter(6,0.05);

output = filter(b2,a2,demod);

% Plot

figure;

subplot(4,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t,am);

title('AM Signal');

xlabel('Time');

ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t,vsb);

title('VSB Modulated Signal');

xlabel('Time');

ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t,output);

title('Demodulated Output');

xlabel('Time');

ylabel('Amplitude');

grid on;

OUTPUT: