GENERATION AND ANALYSIS OF FREQUENCY MODULATED (FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB

Aim:

To generate and analyze Frequency Modulated (FM) and Phase Modulated (PM) signals using MATLAB by varying the instantaneous frequency and phase of a carrier signal in accordance with the message signal.

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;              % Sampling frequency

t = 0:1/Fs:0.02;          % Time vector

fm = 1000;                % Message frequency = 1 kHz

fc = 10000;               % Carrier frequency = 10 kHz

Am = 1;                   % Message amplitude

Ac = 1;                   % Carrier amplitude

kf = 5000;                % FM frequency sensitivity

kp = 2;                   % PM phase sensitivity

m = Am*cos(2*pi*fm*t);

%% 2. Carrier Signal

carrier = Ac*cos(2*pi*fc*t);

% FM equation:

% sFM(t) = Ac*cos(2*pi*fc*t + 2*pi*kf*integral(m(t)))

phase_FM = 2*pi*fc*t + 2*pi*kf*cumsum(m)/Fs;

FM_signal = Ac*cos(phase_FM);

% PM equation:

% sPM(t) = Ac*cos(2*pi*fc*t + kp*m(t))

phase_PM = 2*pi*fc*t + kp*m;

PM_signal = Ac*cos(phase_PM);

N = length(t);

f = (-N/2:N/2-1)*(Fs/N);

FM_fft = abs(fftshift(fft(FM_signal)))/N;

PM_fft = abs(fftshift(fft(PM_signal)))/N;

figure;

subplot(5,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Message Signal');

grid on;

subplot(5,1,2);

plot(t,carrier);

xlabel('Time (s)');

ylabel('Amplitude');

title('Carrier Signal');

grid on;

xlim([0 0.003]);

subplot(5,1,3);

plot(t,FM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Frequency Modulated (FM) Signal');

grid on;

xlim([0 0.003]);

subplot(5,1,4);

plot(t,PM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Phase Modulated (PM) Signal');

grid on;

xlim([0 0.003]);

subplot(5,1,5);

plot(t,m);

hold on;

plot(t,FM_signal);

plot(t,PM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Comparison of Message, FM and PM Signals');

grid on;

xlim([0 0.003]);

legend('Message','FM','PM');

figure;

plot(f,FM_fft);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Frequency Spectrum of FM Signal');

grid on;

xlim([-20000 20000]);

figure;

plot(f,PM_fft);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Frequency Spectrum of PM Signal');

grid on;

xlim([-20000 20000]);

OUTPUT: