PERFORMANCE ANALYSIS OF FM SYSTEM UNDER GAUSSIAN WHITE NOISE USING MATLAB

Aim:

1. To generate a Frequency Modulated (FM) signal in MATLAB using the specified carrier and message signal parameters.

2. To introduce Additive White Gaussian Noise (AWGN) into the generated FM signal and simulate the effect of a noisy communication channel using MATLAB.

3. To analyze the effect of AWGN on the transmitted FM signal by comparing the original and noisy FM signals using time-domain and frequency-domain analysis.

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;              % Sampling frequency

t = 0:1/Fs:0.02;          % Time vector

fm = 1000;                % Message frequency = 1 kHz

fc = 10000;               % Carrier frequency = 10 kHz

Am = 1;                   % Message amplitude

Ac = 1;                   % Carrier amplitude

kf = 3000;                % Frequency sensitivity

SNR_dB = 10;              % Input SNR

m = Am*cos(2*pi*fm*t);

phase = 2*pi*fc*t + 2*pi*kf*cumsum(m)/Fs;

FM_signal = Ac*cos(phase);

signal_power = mean(FM_signal.^2);

SNR_linear = 10^(SNR_dB/10);

noise_power = signal_power/SNR_linear;

noise = sqrt(noise_power)*randn(size(FM_signal));

noisy_FM = FM_signal + noise;

% Hilbert transform to obtain instantaneous phase

analytic_signal = hilbert(noisy_FM);

instantaneous_phase = unwrap(angle(analytic_signal));

% Differentiate phase to obtain instantaneous frequency

instantaneous_frequency = ...

gradient(instantaneous_phase)/(2*pi)*Fs;

% Remove carrier frequency

recovered = instantaneous_frequency - fc;

% Remove DC component

recovered = recovered - mean(recovered);

lpFilt = designfilt('lowpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency', 1500, ...

'SampleRate', Fs);

recovered = filtfilt(lpFilt,recovered);

recovered = recovered / max(abs(recovered));

MSE = mean((m - recovered).^2);

signal_power_message = mean(m.^2);

error_power = mean((m - recovered).^2);

output_SNR = 10*log10(signal_power_message/error_power);

fprintf('---------------------------------------\n');

fprintf('FM PERFORMANCE UNDER GAUSSIAN WHITE NOISE\n');

fprintf('---------------------------------------\n');

fprintf('Message Frequency = %.2f Hz\n',fm);

fprintf('Carrier Frequency = %.2f Hz\n',fc);

fprintf('Frequency Sensitivity = %.2f Hz/V\n',kf);

fprintf('Input SNR = %.2f dB\n',SNR_dB);

fprintf('Output SNR = %.2f dB\n',output_SNR);

fprintf('Mean Square Error = %.6f\n',MSE);

figure;

subplot(4,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Original Message Signal');

grid on;

subplot(4,1,2);

plot(t,FM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Generated FM Signal');

grid on;

xlim([0 0.003]);

subplot(4,1,3);

plot(t,noisy_FM);

xlabel('Time (s)');

ylabel('Amplitude');

title(['FM Signal with Gaussian White Noise (SNR = ', ...

num2str(SNR_dB),' dB)']);

grid on;

xlim([0 0.003]);

subplot(4,1,4);

plot(t,recovered);

xlabel('Time (s)');

ylabel('Amplitude');

title('Recovered Message Signal');

grid on;

OUTPUT: