Performance Evaluation of Digital Communication
Systems Using Eye Diagram Analysis under Additive
White Gaussian Noise (AWGN) Using MATLAB

Aim:

1.To generate a digital communication signal in MATLAB and transmit it through an

Additive White Gaussian Noise (AWGN) channel.

2. To obtain and analyze the eye diagram of the received digital signal for different noise

conditions.

3. To measure eye-opening parameters such as eye height, eye width, crossing level and

timing variation.

4. To study the effect of decreasing SNR / Eb/N0 on inter-symbol interference, noise margin

and timing performance.

5. To compare the transmitted and received signal characteristics and evaluate

communication-system performance.

Program Code:

%% Performance Evaluation of Digital Communication Systems

%% Using Eye Diagram Analysis under AWGN (BPSK + RRC pulse shaping)

%% No toolbox required

clc; clear; close all;

%% Parameters

numBits = 1e5;            % Number of bits

sps     = 8;              % Samples per symbol

span    = 6;              % Filter span in symbols

rolloff = 0.5;            % Roll-off factor

SNR_dB  = [8 5 2 0];      % Eb/N0 values to test (dB)

%% Root Raised Cosine filter (hand-written)

t = (-span*sps/2 : span*sps/2) / sps;

rrc = zeros(size(t));

for i = 1:length(t)

ti = t(i);

if abs(ti) < 1e-10

rrc(i) = 1 - rolloff + 4*rolloff/pi;

elseif abs(abs(ti) - 1/(4*rolloff)) < 1e-8

rrc(i) = (rolloff/sqrt(2)) * ((1+2/pi)*sin(pi/(4*rolloff)) + ...

(1-2/pi)*cos(pi/(4*rolloff)));

else

rrc(i) = (sin(pi*ti*(1-rolloff)) + 4*rolloff*ti*cos(pi*ti*(1+rolloff))) ...

/ (pi*ti*(1-(4*rolloff*ti)^2));

end

end

rrc = rrc(:) / sqrt(sum(rrc.^2));             % Unit energy normalization

%% Transmitter

data    = randi([0 1], numBits, 1);

symbols = 2*data - 1;                         % BPSK mapping (0 -> -1, 1 -> +1)

txUp = zeros(numBits*sps, 1);

txUp(1:sps:end) = symbols;                    % Upsample

txSignal = conv(txUp, rrc);                   % Pulse shaping

delay = span * sps;                           % Total delay of TX + RX filters

%% Channel + Receiver for each SNR

BER = zeros(1, length(SNR_dB));

figure('Name','Eye Diagrams under AWGN','Position',[100 100 1000 700]);

for k = 1:length(SNR_dB)

% AWGN channel (SNR taken as Eb/N0)

EbN0  = 10^(SNR_dB(k)/10);

sigma = sqrt(1/(2*EbN0));                 % Noise std per sample

rxSignal = txSignal + sigma*randn(size(txSignal));

% Matched filter

rxFiltered = conv(rxSignal, rrc);

% Remove filter delays

rxFiltered = rxFiltered(delay+1 : end-delay);

% Downsample to symbol rate

rxSymbols = rxFiltered(1:sps:end);

rxSymbols = rxSymbols(1:numBits);

% Hard decision

rxBits = rxSymbols > 0;

% BER calculation

BER(k) = sum(data ~= rxBits) / numBits;

% Eye diagram (two symbol periods per trace)

subplot(2,2,k);

eyeLen = 2*sps;

numTraces = floor(length(rxFiltered)/eyeLen) - 1;

hold on;

for n = 1:min(numTraces, 200)

idx = (n-1)*eyeLen + 1 : n*eyeLen;

plot(0:eyeLen-1, rxFiltered(idx), 'b');

end

hold off;

grid on;

title(['Eye Diagram, Eb/N0 = ' num2str(SNR_dB(k)) ' dB']);

xlabel('Samples'); ylabel('Amplitude');

ylim([-3 3]);

end

%% Eye opening measurement (noise-free)

cleanFiltered = conv(txSignal, rrc);

cleanFiltered = cleanFiltered(delay+1 : end-delay);

cleanSymbols  = cleanFiltered(1:sps:end);

eyeHeight = min(cleanSymbols(cleanSymbols > 0)) - max(cleanSymbols(cleanSymbols < 0));

fprintf('Ideal eye opening (noise-free): %.3f\n\n', eyeHeight);

%% BER results

theoryBER = 0.5 * erfc(sqrt(10.^(SNR_dB/10)));   % Theoretical BPSK BER

figure;

semilogy(SNR_dB, BER, 'o-r', 'LineWidth', 1.5); hold on;

semilogy(SNR_dB, theoryBER, 's--b', 'LineWidth', 1.5);

grid on;

xlabel('Eb/N0 (dB)'); ylabel('Bit Error Rate');

title('BER vs Eb/N0 under AWGN');

legend('Simulated', 'Theoretical');

fprintf('Eb/N0 (dB)   Simulated BER   Theoretical BER\n');

for k = 1:length(SNR_dB)

fprintf('%6d       %.2e        %.2e\n', SNR_dB(k), BER(k), theoryBER(k));

end

OUTPUT: