Characteristics Study of Side Band Power And Frequencies in AM Using MATLAB

Aim:

To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and modulating signal parameters

Program Code:

clc;

clear;

close all;

% Parameters

Ac = 10;              % Carrier amplitude

Am = 5;               % Message amplitude

fc = 1000;            % Carrier frequency (Hz)

fm = 100;             % Message frequency (Hz)

mu = Am/Ac;           % Modulation index

% Time

fs = 100000;

t = 0:1/fs:0.05;

% AM signal

m = Am*cos(2*pi*fm*t);

c = Ac*cos(2*pi*fc*t);

s = Ac*(1 + mu*cos(2*pi*fm*t)).*cos(2*pi*fc*t);

% Plot message

subplot(3,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Message Signal');

grid on;

% Plot carrier

subplot(3,1,2);

plot(t,c);

xlabel('Time (s)');

ylabel('Amplitude');

title('Carrier Signal');

grid on;

% Plot AM signal

subplot(3,1,3);

plot(t,s);

xlabel('Time (s)');

ylabel('Amplitude');

title('AM Signal');

grid on;

% Sideband frequencies

USB = fc + fm;

LSB = fc - fm;

% Power calculation

Pc = Ac^2/2;

Pusb = (mu^2 * Pc)/4;

Plsb = (mu^2 * Pc)/4;

Ptotal = Pc + Pusb + Plsb;

fprintf('Carrier Frequency = %.2f Hz\n',fc);

fprintf('Lower Sideband Frequency = %.2f Hz\n',LSB);

fprintf('Upper Sideband Frequency = %.2f Hz\n',USB);

fprintf('Carrier Power = %.2f W\n',Pc);

fprintf('LSB Power = %.2f W\n',Plsb);

fprintf('USB Power = %.2f W\n',Pusb);

fprintf('Total Power = %.2f W\n',Ptotal);

OUTPUT: