MATLAB SIMULATION OF SUPERHETERODYNERECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

AIM:
	i) To model and simulate the operation of a superheterodyne receiver for an AM communication system using MATLAB by implementing frequency mixing and intermediate frequency (IF)

ii) To recover the original message signal through intermediate frequency filtering and envelope detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

PROGRAM CODE:

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.05;

fc = 10000;       % Received carrier frequency

fm = 500;         % Message frequency

fLO = 15000;      % Local oscillator frequency

fIF = 5000;       % Intermediate frequency

% Message signal

m = cos(2*pi*fm*t);

% AM received signal

AM = (1 + 0.5*m).*cos(2*pi*fc*t);

% Local oscillator

LO = cos(2*pi*fLO*t);

% Mixer

mixed = AM .* LO;

% IF bandpass filter

[b,a] = butter(6, [(fIF-1500)/(fs/2) (fIF+1500)/(fs/2)], 'bandpass');

IF = filter(b,a,mixed);

% IF demodulation

demod = IF .* cos(2*pi*fIF*t);

% Low pass filter

[b2,a2] = butter(6,1000/(fs/2));

output = filter(b2,a2,demod);

% Plot signals

figure;

subplot(5,1,1);

plot(t,m);

title('Message Signal');

grid on;

subplot(5,1,2);

plot(t,AM);

title('Received AM Signal');

grid on;

subplot(5,1,3);

plot(t,mixed);

title('Mixer Output');

grid on;

subplot(5,1,4);

plot(t,IF);

title('Intermediate Frequency Signal');

grid on;

subplot(5,1,5);

plot(t,output);

title('Demodulated Output');

grid on;

OUTPUT: