CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

Aim:

1. To generate a Frequency Modulated (FM) signal in MATLAB using specified carrier and message signal parameters.

2. To calculate and verify the frequency deviation of the FM signal from the given carrier frequency and modulating signal parameters.

3. To determine and verify the modulation index of the FM signal using the relationship between frequency deviation and message frequency.

4. To analyze the effect of message frequency on frequency deviation and modulation index, and verify their relationship through MATLAB simulation

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;              % Sampling frequency

t = 0:1/Fs:0.02;          % Time vector

fm = 1000;                % Message frequency = 1 kHz

fc = 10000;               % Carrier frequency = 10 kHz

Am = 1;                   % Message amplitude

Ac = 1;                   % Carrier amplitude

kf = 5000;                % Frequency sensitivity (Hz/V)

m = Am*cos(2*pi*fm*t);

% Frequency deviation:

% Delta_f = kf * Am

delta_f = kf * Am;

% Beta = Delta_f / fm

beta = delta_f / fm;

phase = 2*pi*fc*t + 2*pi*kf*cumsum(m)/Fs;

FM_signal = Ac*cos(phase);

instantaneous_phase = unwrap(angle(hilbert(FM_signal)));

instantaneous_frequency = ...

gradient(instantaneous_phase)/(2*pi)*Fs;

max_freq = max(instantaneous_frequency);

min_freq = min(instantaneous_frequency);

measured_delta_f = (max_freq - min_freq)/2;

fprintf('---------------------------------------\n');

fprintf('FM FREQUENCY DEVIATION AND MODULATION INDEX\n');

fprintf('---------------------------------------\n');

fprintf('Carrier Frequency       = %.2f Hz\n', fc);

fprintf('Message Frequency       = %.2f Hz\n', fm);

fprintf('Frequency Sensitivity   = %.2f Hz/V\n', kf);

fprintf('Message Amplitude       = %.2f V\n', Am);

fprintf('\nTheoretical Frequency Deviation = %.2f Hz\n', delta_f);

fprintf('Theoretical Modulation Index    = %.2f\n', beta);

fprintf('\nMeasured Maximum Frequency      = %.2f Hz\n', max_freq);

fprintf('Measured Minimum Frequency      = %.2f Hz\n', min_freq);

fprintf('Measured Frequency Deviation    = %.2f Hz\n', measured_delta_f);

figure;

subplot(3,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Message Signal');

grid on;

subplot(3,1,2);

plot(t,FM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('FM Signal');

grid on;

xlim([0 0.003]);

subplot(3,1,3);

plot(t,instantaneous_frequency);

xlabel('Time (s)');

ylabel('Frequency (Hz)');

title('Instantaneous Frequency of FM Signal');

grid on;

xlim([0 0.005]);

OUTPUT: