POWER SPECTRAL DENSITY (PSD) ANALYSIS OF PHASE MODULATED (PM) SIGNALS USING MATLAB

Aim:

- To generate a Phase Modulated (PM) signal in MATLAB using specified carrier and message signal parameters.

- ii) To compute the frequency spectrum of the PM signal using the Fast Fourier Transform (FFT).

- iii) To calculate and plot the Power Spectral Density (PSD) of the PM signal using MATLAB.

- iv) To identify the carrier component and sideband frequencies present in the PM spectrum.

- v) To analyze the effect of phase modulation on the spectral distribution and bandwidth of the transmitted signal.

- vi)To investigate the influence of modulation index on the Power Spectral Density characteristics of the PM signal.

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;              % Sampling frequency

t = 0:1/Fs:0.02;          % Time vector

fm = 1000;                % Message frequency = 1 kHz

fc = 10000;               % Carrier frequency = 10 kHz

Am = 1;                   % Message amplitude

Ac = 1;                   % Carrier amplitude

kp = 2;                   % Phase sensitivity

m = Am*cos(2*pi*fm*t);

carrier = Ac*cos(2*pi*fc*t);

% PM equation:

% sPM(t) = Ac*cos(2*pi*fc*t + kp*m(t))

PM_signal = Ac*cos(2*pi*fc*t + kp*m);

%% 4. Calculate Power Spectral Density

[PSD,f] = pwelch(PM_signal, [], [], [], Fs);

figure;

subplot(3,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Message Signal');

grid on;

subplot(3,1,2);

plot(t,PM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Phase Modulated (PM) Signal');

grid on;

xlim([0 0.003]);

subplot(3,1,3);

plot(f,10*log10(PSD));

xlabel('Frequency (Hz)');

ylabel('Power/Frequency (dB/Hz)');

title('Power Spectral Density of PM Signal');

grid on;

xlim([0 20000]);

OUTPUT: